VERILOG · LANGUAGE FUNDAMENTALS WIRE; TRI; WAND; WOR; TRI0; TRI1; SUPPLY0. SCALAR; VECTOR; SIGNED; INTEGER; TIME; REAL; REALTIME. Data Types Verilog Data Types Reference & Examples Net and variable-level storage: driven wires, resolved multi-driver nets, trireg-style pull defaults, 4-state registers, and time/real value types. A working reference on all 6 Verilog data type categories and their 14 variants, demonstrated through runnable examples. Covers net types (wire, tri, wand, wor, tri0, tri1, supply0), reg/variable types (scalar, vector, signed), and integer, time, real and realtime data types. File Type Verilog Code Reference Topics 6 data type categories, 14 variants with $display output Includes Runnable modules & example outputs Target RTL & Design Verification Engineers AUTHOR PAYASAMPALLI VENKATA VAMSI github.com/VAMSI-243 · linkedin.com/in/payasampllivamsi.

//=======================================================================

//                                              1.Net Data type

//=======================================================================

//======================================

//                                 a.wire

//======================================

module tb;

wire a;

wire b;

wire y;

assign a = 1'b1;

assign b = 1'b0;

assign y = a & b;

initial begin

#1;

$display("a = %b", a);

$display("b = %b", b);

$display("y = %b", y);

end

endmodule

//output:

# a = 1   ,     # b=0    ,    # y=0;

//======================================

//                          b.tri

//======================================

module tb;

tri y;

assign y = 1'b1;

initial begin

#1;

$display("y = %b", y);

end

endmodule

//output:y=1;

//======================================

//                              c.wand

//======================================

module tb;

wand y;

assign y = 1'b1;

assign y = 1'b0;

initial begin

#1;

$display("y = %b", y);

end

endmodule

//output:y=0

//======================================

//                                d.wor

//======================================

module tb;

wor y;

assign y = 1'b0;

assign y = 1'b0;

initial begin

#1;

$display("y = %b", y);

end

endmodule

//output:y=0

//======================================

//                      e.tri0

//======================================

module tb;

tri0 y;

initial begin

#1;

$display("y = %b", y);

end

endmodule

//output:y=0;

//======================================

//                            f.tri1

//======================================

module tb;

tri1 y;

initial begin

#1;

$display("y = %b", y);

end

endmodule

//output: y=1;

//======================================

//           g.supply0 and supply1

//======================================

module tb;

supply0 gnd;

supply1 vdd;

initial begin

$display("gnd = %b", gnd);

$display("vdd = %b", vdd);

end

endmodule

//output:

//gnd=z;

//vdd=z;

//================================================================

//                                   2.Variable/reg Data Type

//================================================================

//======================================

//                     a.scalar reg

//======================================

module tb;

reg a;

initial begin

a = 1'b1;

$display("a = %b", a);

end

endmodule

//=====================================

//                     b.Vector   reg

//=====================================

module tb;

reg [7:0] data;

initial begin

data = 8'b10101100;

$display("data = %b", data);

$display("data = %h", data);

end

endmodule

//output: data=10101100,data=ac;

//=====================================

//               c.signed reg

//=====================================

module tb;

reg signed [7:0] a;

initial begin

a = -5;

$display("a = %d", a);

end

endmodule

//=======================================================

//                                     3.integer

//=======================================================

module tb;

integer a;

integer b;

integer result;

initial begin

a = 20;

b = 5;

result = a + b;

$display("a      = %0d", a);

$display("b      = %0d", b);

$display("result = %0d", result);

end

endmodule

module tb;

integer i;

initial begin

for(i = 0; i < 5; i = i + 1) begin

$display("i = %0d", i);

end

end

endmodule

//=======================================================

//                                     4.time

//=======================================================

`timescale 1ns/1ns

module tb;

time t1;

time t2;

initial begin

t1 = $time;

#10;

t2 = $time;

$display("Start time = %0t", t1);

$display("End time   = %0t", t2);

end

endmodule

//=======================================================

//                                    5.real

//=======================================================

module tb;

real a;

real b;

real result;

initial begin

a = 10.5;

b = 2.5;

result = a + b;

$display("a      = %f", a);

$display("b      = %f", b);

$display("result = %f", result);

end

endmodule

//=======================================================

//                                     6.realtime

//=======================================================

`timescale 1ns/1ps

module tb;

realtime t;

initial begin

#10.5;

t = $realtime;

$display("Current time = %0.2f ns", t);

end

endmodule