VERILOG · LANGUAGE FUNDAMENTALS DEFINE; UNDEF; INCLUDE; IFDEF; IFNDEF; ELSE; ELSIF. ENDIF; TIMESCALE; DEFAULT_NETTYPE; RESETALL; CELLDEFINE; UNCONNECTED_DRIVE. Compiler Directives Verilog Compiler Directives Reference & Examples Arithmetic-free, preprocessor-level control: macro definition, conditional compilation, file inclusion, timescale, net-type, and cell-boundary directives. A working reference on all 13 Verilog compiler directives, demonstrated through runnable examples. Covers `define/`undef macros, `include file insertion, `ifdef/`ifndef/`else/`elsif/`endif conditional compilation, `timescale simulation units, `default_nettype, `resetall, `celldefine and `unconnected_drive. File Type Verilog Code Reference Topics 13 compiler directives with $display output Includes Runnable modules & example outputs Target RTL & Design Verification Engineers AUTHOR PAYASAMPALLI VENKATA VAMSI github.com/VAMSI-243 · linkedin.com/in/payasampllivamsi/.

//==========================================================

//                                                1.`define

//==========================================================

`define WIDTH 4

module tb;

reg [`WIDTH-1:0] a;

initial begin

a = 4'b1010;

$display("a = %b", a);

end

endmodule

//output:1010

//==========================================================

//                                                2.`undef

//==========================================================

`define WIDTH 4

`undef WIDTH

module tb;

`ifdef WIDTH

initial

$display("WIDTH is defined");

`else

initial

$display("WIDTH is NOT defined");

`endif

endmodule

//output:WIDTH is NOT defined

//==========================================================

//                                                3.`include

//==========================================================

`include "parameter.v"

module tb;

reg [`WIDTH-1:0] a;

initial begin

a = `VALUE;

$display("WIDTH = %0d", `WIDTH);

$display("a = %0d", a);

end

endmodule

//==========================================================

//                                                4.`ifdef

//==========================================================

`define DEBUG

module tb;

`ifdef DEBUG

initial begin

$display("DEBUG mode is ON");

end

`endif

endmodule

//output:DEBUG mode is ON

//==========================================================

//                                                5.`ifndef

//==========================================================

`ifndef WIDTH

`define WIDTH 8

`endif

module tb;

reg [`WIDTH-1:0] a;

initial begin

a = 8'b10101010;

$display("a = %b", a);

end

endmodule

//output:a = 10101010

//==========================================================

//                                         6.`else

//==========================================================

`define DEBUG

module tb;

`ifdef DEBUG

initial

$display("Debug mode");

`else

initial

$display("Normal mode");

`endif

endmodule

//output:# Debug mode

//==========================================================

//                                                7.`elseif

//==========================================================

`define FPGA

module tb;

`ifdef ASIC

initial

$display("ASIC design");

`elsif FPGA

initial

$display("FPGA design");

`else

initial

$display("Unknown design");

`endif

endmodule

//output:FPGA design

//==========================================================

//                                                8.`endif

//==========================================================

`define DEBUG

module tb;

`ifdef DEBUG

initial begin

$display("Debug information");

end

`endif

endmodule

//output:Debug information

//==========================================================

//                                         9.`timescale

//==========================================================

`timescale 1ns/1ps

module tb;

initial begin

#10;

$display($time,"Executed after 10 ns");

end

endmodule

//output:#10 Executed after 10 ns

//==========================================================

//                         10.`default_nettype

//==========================================================

module tb;

wire a;

assign b = a;   // b is automatically treated as wire

endmodule

//==========================================================

//                                     11.`resetall

//==========================================================

`timescale 1ns/1ps

module tb;

initial begin

#10;

$display("Hello");

end

endmodule

`resetall

//output:# Hello

//==========================================================

//              12.`celldefine and `endcelldefine

//==========================================================

`celldefine

module tb(input a, b, output y);

assign y = a & b;

initial

$display("y=%d",y);

endmodule

`endcelldefine

//output: x

//==========================================================

//                   13.`unconnected_drive

//==========================================================

`unconnected_drive pull0

module tb(

input a,

input b,

output y);

assign y = a & b;

endmodule

`nounconnected_drive

//==========================================================

//           EX code Using some directives

//==========================================================

`timescale 1ns/1ps

`default_nettype none

`define WIDTH 8

`define DEBUG

module tb(

input  wire [`WIDTH-1:0] a,

input  wire [`WIDTH-1:0] b,

output wire [`WIDTH-1:0] sum);

assign sum = a + b;

`ifdef DEBUG

initial begin

$display("DEBUG: Adder started");

$display("WIDTH = %0d", `WIDTH);

end

`endif

endmodule

`default_nettype wire

//output:

//DEBUG: Adder started

//# WIDTH = 8

//end